LINE AND TRANSFORMER DATA

(File = LDAT, una línea por rama)

Variable
Columns
Format
Comments

FROM BUS Number
1 - 4
I4


TO BUS Number
5 - 8
I4


Circuit Number
9 - 10
I2


R, X (Positive/Negative Sequence)
11 - 22
F12
Impedance, in percent


23 - 34
F12

Charging (Positive/Negative Sequence)
35 - 46
F12
Percent

Rating
47 - 54
F8
Percent

Minimum Tap, or Minimum Phase Shift Angle
55 - 61
F7
P.U, FROM BUS side

Maximum Tap, or Maximum Phase Shift Angle
62 - 68
F7
P.U, FROM BUS side

Tap Step Size, or Phase Shift Step Size
69 - 75
F7
Per unit, or degrees

Tap
76 - 82
F7
P.U,  FROM BUS side

Phase Shift
83 - 90
F8
Degrees, FROM BUS 

Connection Type for Transformers and Shunt
142 - 143
I2
For transformers:




0 or 1
GY
GY




2
GY
Y




3
Y
GY




4
Y
Y




5





6
GY




7
Y




8

GY




9

Y




For shunt elements:




0 or 1 = Grounded Wye.




Otherwise, ungrounded 

Grounding Impedance R, X
144 - 155
F12


